LMS Software Processing Training
Best Practice for Production Flight

Teledyne Optech Incorporated

Tel:  +1 905 660 0808      Fax:  +1 905 660 0829

Email:  technical.solutions@teledyneoptech.com

This document describes the best practise of data processing when flying a large area.

Flight Planning

Plan the production area first with its optimal PRF, Scan Frequency, FOV and AGL. Draw a cross line or 2 opposing cross lines that goes through all the production lines:

1        2       3       4         5       6         7         8         9       10       11      12         13     14      15

16

17

Day 1                                       Day 2                                             Day 3

- Fly a (3 x 3 + 1) calibration flight over a residential area with the same flight parameters.

LMS Boresight Block

- Process the calibration flight in LMS with the block type as “Boresight + Sensor Polynomial Correction”.

- Accept the calibration result and save the newly created LCP if the conditions below are met:

  - The tie plane error plot in the refined LMS plot is a thin (+/- 5cm) and straight line;

  - The horizontal error (roofline RMS) is smaller than AGL * 1/10000

- Repeat Step 2 for a max of 2 times if the conditions are not met; if the result is still not good, send the calibration data to Optech.

LMS Production Block

There are 2 different scenarios to process a big production area.

- If the whole area contains less than 4 flights and/or if the processing computer is a very powerful one with 64+ CPUs and sufficient SSD storage, process with the “Single-Block” method:

- Create a LMS project and add one mission per flight; for each mission add all the flight lines to ALS lines so this project has 17 ALS lines in total.

- Set Line #16 & 17 as “Cross Line” Versus “Production Line”.

- Create a block and add all the 17 lines in it.

- Choose “Production” block as the block type and finish creating the block.

- Expend the block details and fix the Orientation & Position corrections for Line #16 & 17.

- Run the block.

- Review the Tieplane plot of the refined result to see if it is thin and straight; also review the horizontal offset to see if it meets the specs.

- If the whole area contains more than 4 flights and/or if the processing computer is not that powerful, process with the “Multiple-Block” method:

- Create a LMS project and add Day 1’s data as Mission 01;

- Add Line 1 to 5, and 16, 17 to ALS lines.

- Create a block 01 and select all the 7 lines in it

- Choose “Production” block as the block type and finish creating the block.

- Expend the block details and fix the Orientation & Position corrections for Line #16 & 17.

- Run the block.

- Review the Tieplane plot of the refined result to see if it is thin and straight; also review the horizontal offset to see if it meets the specs.

- Add Day 2’s data as Mission 02;

- Add Line 6 to 10 to ALS lines;

- Create a new block 02 and select Line 6 to 10 from Day 2.

- Select Line #16 and 17 as the “Control Lines”, so LMS will use the refined results of these 2 cross lines as the control points and restrain the calibration results of Block 02.

- Choose “Production” block as the block type and finish creating the block.

- Run the block and later review the Tieplane plot of the refined result to see if it is thin and straight; also review the horizontal offset to see if it meets the specs.

- Repeat Step 8 to 13 for Day 3.

- Review the overlapping areas between Day 1, 2, and 3; since the 3 blocks have the same cross lines as control, the overlapping areas should have no misalignments.